PID_ValveController_Simple Integration Guide

Purpose

This package is a Quick build of a simplified LabVIEW FPGA PID controller intended for CLIP or IP Integration Node style import. It removes Alpha, Beta, Gamma, and Linearity to keep the interface simple for basic tank, valve, or flow control use. Assumptions follow the single-file generation guide: cRIO-class FPGA target, 40 MHz master clock, and 5 MHz functional update rate via Ce. [file:131]

Build mode

- Build mode: Quick

- Validation level: structural generation only in this pass; not a full validated build

- Test document: not generated because only Quick was requested [file:131]

Clocking

- Clk: master FPGA clock, recommended 40 MHz

- Ce: clock-enable pulse, recommended 1 pulse every 8 clocks for 5 MHz effective update

- aReset: active-high reset for internal state [file:131]

Terminal datatype table

| Terminal | Direction | Description | LabVIEW datatype | HDL type | Notes |
| --- | --- | --- | --- | --- | --- |
| Clk | In | Master FPGA clock | Boolean | std_logic | Map on Clock and Enable Signals page |
| Ce | In | Functional update enable | Boolean | std_logic | Recommended for sequential IP |
| aReset | In | Active-high state reset | Boolean | std_logic | Keep as Boolean data/reset input |
| Setpoint | In | Desired process setpoint | FXP<+/-,32,16> | std_logic_vector(31 downto 0) | Engineering-unit style value |
| ProcessVariable | In | Measured process value | FXP<+/-,32,16> | std_logic_vector(31 downto 0) | Engineering-unit style value |
| SetpointHigh | In | Upper setpoint clamp | FXP<+/-,32,16> | std_logic_vector(31 downto 0) |  |
| SetpointLow | In | Lower setpoint clamp | FXP<+/-,32,16> | std_logic_vector(31 downto 0) |  |
| OutputHigh | In | Upper output clamp | FXP<+/-,32,16> | std_logic_vector(31 downto 0) |  |
| OutputLow | In | Lower output clamp | FXP<+/-,32,16> | std_logic_vector(31 downto 0) |  |
| ProportionalGainKc | In | Proportional gain | FXP<+/-,32,8> | std_logic_vector(31 downto 0) | Gain/rate style datatype |
| IntegralTimeTiMin | In | Integral time in minutes | FXP<+/-,32,16> | std_logic_vector(31 downto 0) | Converted internally |
| DerivativeTimeTdMin | In | Derivative time in minutes | FXP<+/-,32,16> | std_logic_vector(31 downto 0) | Converted internally |
| DtS | In | Sample period in seconds | FXP<+/-,32,8> | std_logic_vector(31 downto 0) | Clamped to minimum internally |
| AutoMode | In | Auto/manual selector | Boolean | std_logic | TRUE = PID auto |
| ManualControl | In | Manual output command | FXP<+/-,32,16> | std_logic_vector(31 downto 0) | Used when AutoMode=0 |
| Reinitialize | In | Logical PID state reset | Boolean | std_logic | Clears I/D state |
| Output | Out | Final controller output | FXP<+/-,32,16> | std_logic_vector(31 downto 0) | Saturated output |
| DtOut | Out | Sanitized dt used internally | FXP<+/-,32,8> | std_logic_vector(31 downto 0) | Debug/visibility |

Import notes

Clock and Enable Signals page

- Map Clk as the IP clock.

- Map Ce as the IP enable.

- Use No Derived Clock unless intentionally running a different domain. [file:131]

IP Terminals page

- For Q16.16 terminals, choose FXP, Signed, Integer Word Length = 16.

- For Q8.24 terminals, choose FXP, Signed, Integer Word Length = 8.

- For Boolean terminals, choose Boolean. [file:131]

Suggested starting values for tank control

- Setpoint, ProcessVariable, Output: 0 to 100

- SetpointLow = 0

- SetpointHigh = 100

- OutputLow = 0

- OutputHigh = 100

- ProportionalGainKc: start in the 0.1 to 10 range

- IntegralTimeTiMin: start in the 0.1 to 10 minute range

- DerivativeTimeTdMin: start at 0 for a basic tank loop

- DtS: start around 0.01 s to 0.1 s

Notes

This quick package is intended to be easy to import and configure. It is not yet marked as a fully validated build; if you want that next, the next pass should add a test plan and run the XML/VHDL/LabVIEW preflight steps described in the single-file toolkit guide. [file:131]