MovingAverage CLIP Node — Integration Guide

Algorithm Description

This CLIP node implements a 16-sample sliding-window moving average filter.
Each enabled clock cycle the oldest sample is dropped, the newest is added,
and the running accumulator is multiplied by the pre-scaled gain AvgGain = 1/16
to produce the averaged output with no runtime division on the FPGA.

Generated Files

| File | Purpose |
| --- | --- |
| MovingAverage.vhd | Synthesizable VHDL-2008 entity and RTL architecture |
| MovingAverage.xml | LabVIEW FPGA CLIP Declaration, FormatVersion 4.2 |
| MovingAverage_Integration_Guide.md | This file |
| MovingAverage_Test_Plan.md | Verification and compile-readiness tests |
| MovingAverage_RT_Prescaling.md | RT-side coefficient computation |

Build Mode and Applied Defaults

| Default | Value |
| --- | --- |
| Build mode | Quick Build (files 1–5) |
| Target FPGA | xc7k70tfbg676-1 (NI cRIO-class Kintex-7) |
| Master clock | 40 MHz |
| VHDL standard | VHDL-2008 |
| Window size N | 16 samples |
| Input/Output FXP | FXP<+/-,32,16> signed, IWL=16, FWL=16, LSB≈1.526e-5 |
| AvgGain FXP | FXP<+/-,32,8> signed, IWL=8, FWL=24, LSB≈5.96e-8 |
| Reset | aReset, active-high, synchronous |
| Enable | Ce, active-high |
| Closed-loop test | None |

Terminal Mapping Table

| Front Panel Label | VHDL Port | Direction | FXP Format | Range | Notes |
| --- | --- | --- | --- | --- | --- |
| Clk | Clk | ToCLIP | std_logic (clock) | 40 MHz | Wire to SCTL clock reference |
| aReset | aReset | ToCLIP | Boolean | 0/1 | High clears all state synchronously |
| Ce | Ce | ToCLIP | Boolean | 0/1 | Low freezes filter; high enables update |
| Data In (0-100%) | DataIn | ToCLIP | FXP<+/-,32,16> | 0.0–100.0 | Raw process signal in percent |
| Avg Gain (1/N) | AvgGain | ToCLIP | FXP<+/-,32,8> | ~0.0625 for N=16 | Computed on RT; write once at startup |
| Data Out (0-100%) | DataOut | FromCLIP | FXP<+/-,32,16> | 0.0–100.0 | Averaged output |
| Overflow | Overflow | FromCLIP | Boolean | 0/1 | High if accumulator exceeds guard bits |

LabVIEW FPGA Import Procedure

- In the LabVIEW FPGA project, right-click the FPGA target and select
Properties → Component-Level IP.

- Click Add and browse to MovingAverage.xml.
Confirm the Declaration Name shows MovingAverage with no red X.
(If a red X appears, verify MovingAverage.vhd is in the same directory
as the .xml file.)

- Click OK. The IP Integration Node (MovingAverage) will appear in the
block diagram palette under Component-Level IP.

- Place the node on the FPGA block diagram inside a
Single-Cycle Timed Loop (SCTL).

- Wire terminals:

  - Clk → SCTL clock reference terminal

  - Ce → SCTL Loop Tick output (or wire a TRUE constant for always-on)

  - aReset → your system reset Boolean

  - Data In (0-100%) → FXP control or upstream data source

  - Avg Gain (1/N) → FPGA host DMA or front-panel FXP control

  - Data Out (0-100%) → FXP indicator or downstream node

  - Overflow → Boolean indicator

- IP Terminals page (critical — skip this and constants become 32,32):
For every FXP terminal, right-click the node → IP Terminals:

  - Data In (0-100%) and Data Out (0-100%): Signed=true, WordLength=32, IntegerWordLength=16

  - Avg Gain (1/N): Signed=true, WordLength=32, IntegerWordLength=8

- Compile. Expected resource usage on xc7k70tfbg676-1: ~18 FFs, ~1 DSP48 slice.

Pipeline Latency

| Stage | Latency (cycles) |
| --- | --- |
| Input latch + shift register update | 1 |
| Accumulator update | 1 |
| Gain multiply + shift | 1 |
| Output register | 1 |
| Total end-to-end | 4 clock cycles at 40 MHz = 100 ns |

DataOut reflects the average of the 16 samples ending 4 cycles after DataIn is presented
with Ce high.

Ce Rate vs. Master Clock

The CLIP operates at the full 40 MHz master clock. Ce should be driven
by your loop tick at whatever rate the algorithm needs to sample (e.g., 1 kHz,
10 kHz). When Ce is low the shift register and accumulator are frozen; the
node consumes clock power but produces no state change. DataOut holds its
last value.

What NOT to Do

- Do not write a runtime 1/N divide inside the SCTL. Always pre-compute
AvgGain on the RT side and write it once at startup (or whenever N changes).

- Do not drive 
Ce with a fast free-running signal if your process runs slowerthan the master clock. Use the SCTL loop tick to pace the filter correctly.

- Do not leave AvgGain undriven. An undriven FXP terminal defaults to 0,
producing a zero output regardless of input.

- Do not set FXP terminal formats after placing constants. Set IP Terminals
first, then place controls and wire them.

Pre-Compile Checklist

- [ ] MovingAverage.vhd and MovingAverage.xml are in the same directory

- [ ] CLIP Declaration Name shows MovingAverage with no red X

- [ ] IP Terminals set: DataIn/DataOut IWL=16, AvgGain IWL=8

- [ ] Clk wired to SCTL clock reference

- [ ] Ce wired to SCTL loop tick or TRUE constant

- [ ] aReset wired to system reset Boolean

- [ ] Avg Gain (1/N) written from RT on startup (value ≈ 0.0625 for N=16)

- [ ] Overflow indicator monitored during initial test

- [ ] No red broken wires on FPGA block diagram

- [ ] Run GHDL analysis before Vivado compile: ghdl -a --std=08 MovingAverage.vhd